4WD Autonomous Patrol Car

with Mobile Phone Detection

— Project Technical Report —

Egypt University of Informatics

Department of Data Science and Artificial Intelligence

2025 – 2026

1. INTRODUCTION

This report documents the design, implementation, and evaluation of a 4-wheel-drive (4WD) autonomous patrol robot capable of detecting a person holding a mobile phone using computer vision and deep learning. Upon a confirmed detection, the system triggers a multi-modal alarm consisting of alternating LED lights and an active buzzer, and performs real-time proximity-based obstacle avoidance using an ultrasonic sensor. The entire pipeline runs on-device on a Raspberry Pi 4B — no cloud connectivity is required.

1.1 Motivation

With the growing deployment of embedded AI at the edge, there is increasing demand for low-power autonomous systems capable of performing non-trivial perception tasks in real time. This project explores how modern object detection models (YOLO11s in NCNN format) can be deployed on a resource-constrained single-board computer while simultaneously managing motor control, sensor polling, servo sweep, and a live MJPEG monitoring stream.

1.2 Project Scope

Autonomous forward/backward patrol with timed direction switching

Real-time detection of a person holding a mobile phone via YOLO11s + spatial filtering

Multi-modal alarm: alternating red/blue LEDs at 15 Hz and active buzzer

Proximity avoidance: reverse-then-reposition manoeuvre triggered within 30 cm

Camera pan sweep via SG90 servo on hardware PWM to widen the field of view

Live MJPEG HTTP stream on port 8080 for remote monitoring

2. HARDWARE DESIGN

2.1 Component List

The table below lists every hardware component used in the system.

| Component | Specification / Model | Quantity | Role |
| --- | --- | --- | --- |
| Raspberry Pi 4B | 4 GB RAM, Ubuntu 24.04 LTS | 1 | Main compute & control unit |
| Acrylic 4WD Chassis | Standard 4WD acrylic frame | 1 | Mechanical platform |
| TT DC Motors | Standard TT gear motors | 4 | Drive wheels |
| Wheels | 65 mm standard rubber wheels | 4 | Traction |
| L298N Motor Driver | Dual H-bridge, 2A per channel | 1 | Motor PWM control |
| Pi Camera Module | CSI interface, MJPEG output | 1 | Visual input for detection |
| HC-SR04 Ultrasonic | 2 cm – 400 cm range | 1 | Proximity / obstacle avoidance |
| SG90 Servo Motor | 180° rotation, hardware PWM | 1 | Camera pan sweep |
| 8.4 V Battery | 6000 mAh Li-ion pack | 1 | Main power supply |
| 1 kΩ Resistor | 1/4 W carbon film | 1 | Voltage divider (HC-SR04 ECHO) |
| 2 kΩ Resistor | 1/4 W carbon film | 1 | Voltage divider (HC-SR04 ECHO) |
| Red LED | 5 mm standard LED | 1 | Alarm indicator (police light) |
| Blue LED | 5 mm standard LED | 1 | Alarm indicator (police light) |

2.2 GPIO Pin Assignment

All GPIO references use BCM numbering. The Raspberry Pi 4B operates at 3.3 V logic; any 5 V signal from peripheral sensors must be level-shifted before reaching a GPIO pin.

| Subsystem | Component | GPIO (BCM) | Interface |
| --- | --- | --- | --- |
| Left motors | L298N — ENA (PWM) | GPIO 18 | lgpio PWM |
| Left motors | L298N — IN1 | GPIO 23 | Digital out |
| Left motors | L298N — IN2 | GPIO 24 | Digital out |
| Right motors | L298N — ENB (PWM) | GPIO 13 | lgpio PWM |
| Right motors | L298N — IN3 | GPIO 27 | Digital out |
| Right motors | L298N — IN4 | GPIO 22 | Digital out |
| Camera pan | SG90 Servo (signal) | GPIO 12 | Hardware PWM (PWM0) via sysfs |
| Alarm | Red LED | GPIO 17 | Digital out |
| Alarm | Blue LED | GPIO 25 | Digital out |
| Alarm | Buzzer | GPIO 5 | Digital out |
| Proximity | HC-SR04 — TRIG | GPIO 4 | Digital out |
| Proximity | HC-SR04 — ECHO | GPIO 6 | Digital in (via divider) |

2.3 Voltage Divider — HC-SR04 ECHO Line

The HC-SR04 ultrasonic sensor outputs a 5 V logic signal on its ECHO pin, which would damage the 3.3 V GPIO of the Raspberry Pi if connected directly. A resistor voltage divider is used to bring the signal into a safe range:

R1 = 1 kΩ (between ECHO pin and GPIO 6)

R2 = 2 kΩ (between GPIO 6 and GND)

This produces V_out = 5 V × (2k / (1k + 2k)) ≈ 3.33 V, which is within the safe operating range of the Raspberry Pi GPIO input.

2.4 Servo — Hardware PWM Configuration

Software PWM generated by lgpio caused visible jitter in the SG90 servo whenever the YOLO inference thread loaded the CPU. The issue was eliminated by switching to hardware PWM via the Linux sysfs interface:

dtoverlay=pwm,pin=12,func=4 added to /boot/firmware/config.txt

A udev rule and boot-time setup script auto-export and chmod the sysfs PWM files

The servo thread uses a drift-corrected 20 Hz loop for smooth, jitter-free sweep

3. SOFTWARE ARCHITECTURE

The software stack is built on ROS2 Jazzy using an ament_python package named car_control. The entire system runs as a single ROS2 node (patrol) in one Python file. No ROS topics or services are used internally — all subsystems share Python state protected by threading.Lock objects, keeping latency low and eliminating serialization overhead.

3.1 Thread Architecture

Six concurrent threads handle the different subsystems:

| Thread | Rate | Responsibility |
| --- | --- | --- |
| _camera_reader_loop | Max pipe drain rate | Reads MJPEG from rpicam-vid subprocess; decodes JPEGs; keeps only the latest frame |
| _inference_loop | ~1–2 fps | CLAHE enhancement → YOLO11s NCNN inference → detection logic |
| _sonar_loop | 20 Hz | HC-SR04 pulse measurement; updates shared distance reading |
| _servo_loop | 20 Hz | Drift-corrected sweep of SG90 servo (0° → 180° → 0°) |
| _stream_server_loop | On demand | Serves MJPEG HTTP stream on port 8080 for live monitoring |
| ROS control timer | 20 Hz | State machine: SCANNING / LOCKED transitions, LEDs, buzzer, motor commands |

3.2 State Machine

The node operates a two-state finite state machine governing all system behaviour:

| State | Entry Condition | Behaviour | Exit Condition |
| --- | --- | --- | --- |
| SCANNING | Default / startup | Patrol forward/backward on timed pattern; servo sweeps 0°–180°; inference runs continuously | Person + phone confirmed in same frame |
| LOCKED | Confirmed detection | Motors stop; LEDs flash alternating red/blue at 15 Hz; buzzer on; inference pauses; sonar monitors proximity | 8-second detection window expires with no new confirmation |

4. DETECTION PIPELINE

4.1 Camera Acquisition

rpicam-vid is launched as a subprocess outputting raw MJPEG to stdout at 640×480 resolution at 10 fps. Camera flags --gain 4 --exposure long are used to improve sensitivity in low-light conditions. The camera reader thread drains the pipe continuously, discarding stale frames and always exposing the freshest decoded image to the inference thread.

4.2 Preprocessing — CLAHE

Before inference, CLAHE (Contrast Limited Adaptive Histogram Equalization) is applied to the L channel of the image in LAB colour space. This technique enhances local edge contrast in dim or unevenly lit conditions without over-brightening regions that are already well exposed. The result is a more consistent input distribution for the YOLO model regardless of ambient lighting.

4.3 Object Detection Model

The detection model is YOLO11s exported in NCNN format. NCNN was selected because PyTorch cannot be installed on the Raspberry Pi 4B (the CUDA torch build raises a SIGILL on the ARM Cortex-A72 CPU). The model was exported on a Mac using the ultralytics library with pnnx, then transferred to the Pi via rsync.

Inference parameters:

| Parameter | Value | Rationale |
| --- | --- | --- |
| conf | 0.10 | Low global threshold; per-class gates applied in post-processing |
| iou | 0.45 | Standard NMS overlap threshold |
| agnostic_nms | True | Merge overlapping boxes across classes |
| classes | [0, 67] | COCO class 0 = person; class 67 = cell phone |
| NumPy version | Pinned 1.26.4 | NumPy 2.x causes SIGILL on Cortex-A72 |

4.4 Detection Logic — Spatial Filter

A raw bounding box match is not sufficient for a confirmed detection. All three of the following conditions must be satisfied simultaneously:

A person bounding box is present with confidence ≥ 0.40

A cell phone bounding box is present with confidence ≥ 0.12

The centre point of the phone box falls within the person box expanded by a 25% margin on each side

The spatial containment check eliminates false positives caused by phones visible in the background that are not associated with the detected person. The asymmetric confidence thresholds (0.40 for person, 0.12 for phone) reflect the observation that YOLO11s tends to score handheld phones in the range 0.10–0.25 due to partial occlusion by the hand.

4.5 CPU Management During Alarm

When the system transitions to LOCKED state, the inference thread skips all YOLO processing and idles at 100 ms sleep intervals. This reduces inference CPU usage from approximately 300% to near zero, freeing CPU cores for the sonar busy-wait loop and servo control — both of which are timing-critical.

5. PROXIMITY AVOIDANCE

While the system is in LOCKED state, the sonar loop runs at 20 Hz, continuously updating the shared distance reading. If an object is detected within 30 cm, the following avoidance manoeuvre is executed:

| Step | Action | Duration |
| --- | --- | --- |
| 1 | Motors immediately reverse at duty cycle 60 | Begins instantly |
| 2 | Three short beeps fire from the buzzer simultaneously with reverse start | ~0.4 s |
| 3 | Reverse continues | 1.0 s total |
| 4 | Motors drive forward at the same speed to return to pre-alert position | 1.0 s |
| 5 | Motors stop; LED/buzzer alarm resumes until 8-second detection window expires | Remainder of window |

6. KEY ENGINEERING CHALLENGES

The following table summarises the main technical problems encountered during development, their root causes, and the solutions implemented.

| Problem | Root Cause | Solution |
| --- | --- | --- |
| Servo jitter | lgpio software PWM jitter under YOLO CPU load | Hardware PWM via sysfs; dedicated servo thread |
| Servo slow during inference | Servo driven from ROS timer thread starved by YOLO | Moved servo to independent drift-corrected 20 Hz thread |
| Sonar latency (2–3 s) | YOLO at 300% CPU pre-empts sonar busy-wait, misses echo pulses | Stop inference during LOCKED state, freeing CPU for sonar |
| Phone not detected | Default confidence 0.35 too high; phone scores 0.10–0.25 when held | Per-class filtering: global conf=0.10, phone gate at 0.12 |
| Low-light detection failures | Camera using conservative default gain/exposure settings | --gain 4 --exposure long combined with CLAHE preprocessing |
| Patrol restart after alarm | _patrol_start reset to current time on unlock, losing elapsed progress | Save _patrol_elapsed_at_lock on entry to LOCKED; restore on exit |
| SIGILL on model export | CUDA torch build incompatible with ARM Cortex-A72 | Export on Mac with ultralytics/pnnx/ncnn; rsync to Pi |
| PWM permission on boot | sysfs files are root-only; udev rule quotes stripped by echo/heredoc | Setup script and udev rule written via Python to preserve quotes |
| HC-SR04 damaging GPIO | ECHO pin outputs 5 V, exceeding Pi 3.3 V GPIO limit | 1 kΩ/2 kΩ resistor voltage divider reduces signal to 3.33 V |

7. TUNABLE PARAMETERS

All tunable values are exposed through the ROS2 launch file (patrol.launch.py) and can be overridden at runtime without modifying source code.

| Parameter | Default Value | Description |
| --- | --- | --- |
| base_speed | 45 | Patrol motor duty cycle (0–100) |
| servo_step | 4.0 °/tick | Degrees per servo tick at 20 Hz → 80°/s sweep rate |
| patrol_time | 5.0 s | Duration of each forward/backward patrol segment |
| unlock_secs | 8.0 s | How long the alarm stays active after the last confirmed detection |
| proximity_cm | 30.0 cm | Sonar distance threshold triggering obstacle avoidance |
| led_flash_hz | 15 Hz | LED alternation frequency during alarm state |
| model_path | /home/mohanad/yolo11s_ncnn_model | NCNN model directory path on the Pi |

8. SYSTEM INTEGRATION & TESTING

8.1 Deployment Procedure

1. Model export (Mac): Export YOLO11s to NCNN format using ultralytics + pnnx on a Mac. Transfer the ncnn_model directory to the Pi via rsync.

2. Boot configuration: Add dtoverlay=pwm,pin=12,func=4 to /boot/firmware/config.txt. Install and enable the udev rule and sysfs setup script for hardware PWM.

3. ROS2 workspace build: Clone the car_control package into the ROS2 workspace, run colcon build, and source the install overlay.

4. Launch: ros2 launch car_control patrol.launch.py — all six threads start automatically.

5. Monitoring: Open a browser to http://<pi-ip>:8080 to view the live annotated MJPEG stream.

8.2 Observed System Behaviour

During testing, the following performance characteristics were observed:

Detection latency from phone appearing in frame to LOCKED state: approximately 0.5–1.5 seconds depending on pose and lighting

False positive rate reduced significantly by the spatial containment filter (phone centre inside expanded person box)

Servo jitter eliminated after migration to hardware PWM; sweep is smooth even under full inference CPU load

Sonar response time improved from 2–3 s to near-instant after inference is paused in LOCKED state

Live MJPEG stream usable at ~640×480 with bounding box overlays visible for remote debugging

9. CONCLUSION

This project successfully demonstrates a fully autonomous patrol robot capable of running a complete perception-action loop entirely on edge hardware. Key achievements include:

Deployment of YOLO11s in NCNN format on a Raspberry Pi 4B without PyTorch or cloud dependencies

A multi-threaded ROS2 architecture that decouples camera acquisition, inference, sonar, servo, streaming, and control into concurrent threads

A spatial containment filter that significantly reduces false positives in the phone-detection pipeline

Hardware PWM for jitter-free servo control even under peak CPU load

A clean state machine governing SCANNING and LOCKED modes with configurable parameters

The system demonstrates that resource-constrained single-board computers can host non-trivial AI perception pipelines alongside real-time hardware control when careful attention is paid to thread design, CPU budgeting, and hardware-level peripherals such as PWM and voltage dividers. Future work could extend the system with Wi-Fi telemetry, multi-camera support, or an expanded detection taxonomy.